SmartEnergy Lab Load Simulation Module

Detailed Feature Request: Device Layer + Human/Event Behavior Layer

Prepared for implementation in the SmartEnergy Lab bachelor thesis project

| Revision point | Final decision |
| --- | --- |
| Module boundary | This is explicitly a LOAD module. Final required output is total_power_draw_w. |
| Cadence | Base simulation runs at 1-minute cadence. Long history stores selected 15-minute points. |
| Energy aggregation | Each stored 15-minute point includes momentary power at timestamp and energy consumed during the previous 15 minutes in Wh. |
| Sub-minute display | Frontend life-view may use interpolation/jitter below 1 minute. Sub-minute values are visual only. |
| Professor holidays | Each professor uses exactly 14 holiday days per year. They may be one long block or split into 3-4 smaller breaks. |
| Inverter limit | The 2 kW inverter limit does not apply when grid is available because inverter works in pass-through mode. |

# 1. Purpose and strict scope

The load simulation module generates realistic laboratory electrical consumption for SmartEnergy Lab. It does not decide whether the simulated system can serve this load. It does not execute controller strategy, battery protection, or inverter enforcement. It only calculates what the laboratory equipment actually tries to consume at each simulated moment based on people, devices, events, and external conditions.

total_power_draw_w(t) = always_on_power_w + sum(active_device_power_w at timestamp t)

Final primary output: total_power_draw_w.

The module may read grid availability, planned outage information, battery SoC, weather, sunrise/sunset, weekday and calendar information.

Those inputs only modify human/device behavior: what people decide to turn on. They are not controller decisions.

No served_load_w/requested_load_w split in this module. If a later controller needs such split, it must be built outside this module.

No hard 2 kW clipping when grid is available. Grid pass-through means the laboratory may consume above 2 kW from the grid.

# 2. Inputs and outputs

| Category | Fields / concept | Use inside load module |
| --- | --- | --- |
| Time | timestamp, date, weekday, season | Generate presence intervals, classes, visits, lighting context and event timing. |
| Grid state | grid_available_now, next_outage_start/end, planned outage today | Changes behavior probabilities: people avoid heavy events during outage or just before outage. |
| Battery state | battery_soc_percent, optional battery mode summary | Changes behavior during outage/planned outage. Low SoC suppresses optional loads. |
| Weather/daylight | weather label, rain/cloud/fog, sunrise/sunset, dark_condition | Controls desk lamps and ceiling lamp probability. |
| Device layer | device definitions: power ranges and behavior type | Defines static power ranges used by generated events. |
| Seed/config | simulation_seed, station timezone, class schedule | Makes generated history deterministic and repeatable. |

| Output | Meaning | Required? |
| --- | --- | --- |
| total_power_draw_w | Momentary total power draw at the timestamp after behavior modifiers are applied. | Yes |
| energy_wh_last_15m | Total energy consumed during the previous 15 minutes, computed from 1-minute samples. | Yes for 15-minute persisted rows |
| active_people_count | Professors/students present. Useful for debugging/dashboard, not core physics. | Optional |
| active_event_tags | Tags such as professor_workstation, student_class, kettle_boil, dark_condition. | Optional |
| active_device_breakdown_json | Compact debug list of active device groups/top contributors. | Optional |

# 3. Time cadence, history storage, and live buffer logic

Internal simulation cadence is 1 minute. Device events are resolved at minute granularity.

Long-term history should not store every minute forever unless storage cost is acceptable. Long history stores selected 15-minute points.

Each persisted 15-minute row must include both momentary power at the row timestamp and total energy Wh consumed during the previous 15-minute interval.

Battery drain later should use energy_wh_last_15m, not only momentary_power_w, because short events such as kettle/heat gun/drill would otherwise be misrepresented.

Recent buffer should follow the same idea as existing solar/grid widgets: more detailed data for last 30 minutes, 3 hours, 12 hours, and 24 hours; longer history uses coarser stored/selected points.

Sub-minute frontend “live” view can be generated by interpolation/jitter between 1-minute points. These sub-minute values are visual only and should not be persisted as real simulation data.

energy_wh_last_15m = sum(power_w_for_each_minute / 60.0 for the last 15 one-minute samples)

# 4. Device layer

The device layer is static inventory. It defines what can consume electricity and what power ranges should be used. Behavior rules decide when these devices are active.

| Group | Device | Count | Standby W | Typical W | Active W | Peak W | Behavior notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Critical Internet | ONU / optical terminal | 1 | 5 | 6 | 7 | 8 | Always on baseline. |
| Critical Internet | MikroTik-like router | 1 | 8 | 10 | 12 | 12 | Always on baseline. |
| Critical Internet | 8-10 port Ethernet switch | 1 | 3 | 4 | 5 | 6 | Always on baseline. |
| Workstations | Monoblock / AIO PC | 5 | 5 | 45 | 65 | 90 | Active only when assigned professor/person works at workstation. |
| Workstations | LED desk lamp | 5 | 0 | 6 | 6 | 7 | Binary on/off; depends on person + darkness/weather. |
| Room lighting | Big LED ceiling/panel lamp | 2 | 0 | 40 | 40 | 45 | Shared; mostly dark/soldering condition. |

| Group | Device | Count | Standby W | Typical W | Active W | Peak W | Behavior notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Soldering table | Soldering station | 6 | 3 | 25 | 60 | 75 | Thermostatic: heat-up then holding. Do not model as constant 75 W. |
| Soldering table | Soldering desk lamp | 6 | 0 | 6 | 6 | 7 | Binary on/off; tied to soldering/debug work and darkness. |
| Soldering table | Lab PSU | 6 | 2 | 10 | 30 | 200 | Usually low-power electronics; rare higher draw. |
| Soldering table | Hot glue gun | 6 | 0 | 20 | 40 | 60 | Short thermostat events during hardware/soldering work. |
| Soldering table / workstations | Phone charger | up to 11 | 0 | 5 | 12 | 20 | Per-person random event. |
| High-power | Hand drill | 1 | 0 | 0 | 400 | 450 | Rare short maintenance/tool event. |
| High-power | Heat gun | 1 | 0 | 0 | 1500 | 1500 | Rare short soldering/hardware event. |
| High-power | Kettle | 1 | 0 | 0 | 1200 | 1200 | Shared coffee/tea event; human-triggered. |

| Laptop profile | Probability | Outlet draw if plugged | Plugged probability when grid available | Behavior notes |
| --- | --- | --- | --- | --- |
| MacBook / efficient laptop | 20-25% | 10-30 W | 25-40% | Often runs on battery; low outlet draw. |
| Business laptop | 50-60% | 25-65 W | 50-70% | Normal student laptop profile. |
| Gaming laptop | 20-25% | 90-230 W | 70-90% | High load; short battery life; visible spikes. |

# 5. Human model overview

There are 5 persistent professor/worker agents tied to 5 workstation places.

There are 6 soldering workplaces used by scheduled student classes, random student visits, and occasional professor soldering/equipment work.

People create events. Events activate devices. No people means only always-on critical internet load remains.

Professors have absence states: illness, holidays, field work. If absent, their workstation is off and they cannot initiate any event.

Students are mainly event-based agents: scheduled classes and random visits.

# 6. Professor absence and presence rules

| Rule | Specification |
| --- | --- |
| Illness | 0-2 illness episodes per year per professor. Duration 2-14 consecutive days. Use TNorm(5, 3, 2-14). Slight winter bias optional. |
| Holiday annual budget | Exactly 14 holiday days per professor per year. These 14 days may be taken as one 14-day block or split into 3-4 smaller breaks. |
| Holiday split probability | Suggested: 40% one 14-day block; 35% three blocks totaling 14 days; 25% four blocks totaling 14 days. Blocks should not overlap illness/field work. |
| Field work | Duration 1-3 days. Higher probability Mar-Oct. Suggested daily chance: Nov-Feb 0.5-1.5%; Mar-Oct 2-5%. |
| Absence priority | ill / holiday / field_work override daily presence. If active, the professor does not generate workplace/device events. |

| Profile | Weekday presence | Weekend presence | Arrival/duration | Behavior emphasis |
| --- | --- | --- | --- | --- |
| Regular professor | 85-95% if not absent | 2-8% | Arrival 08:30-11:00; duration 5-9 h | Mostly workstation; occasional soldering/equipment. |
| Young enthusiast professor | 90-98% if not absent | 25-45% | May work late; rare overnight 1-4% | Higher weekend/late/soldering probability. |

# 7. Professor activity state machine

Inside a presence interval, each professor changes activity state. This creates dynamic behavior without hardcoded morning/evening load profiles.

| State | Meaning | Devices potentially activated |
| --- | --- | --- |
| workstation_work | Professor works at assigned monoblock. | Monoblock, workstation lamp, phone charger. |
| soldering_work | Professor uses available soldering workplace/equipment. | Soldering station, soldering lamp, lab PSU, hot glue, rare heat gun/drill. |
| short_absence | Professor is away from room but still in daily presence interval. | Usually no personal devices except maybe monoblock standby. |
| coffee_break | May participate in shared kettle event. | Kettle only if shared event generated. |
| consultation | Low-power discussion/work with student/colleague. | Maybe laptop/monoblock, phone charger, lamp if dark. |
| idle_present | Present but not actively consuming much. | Minimal workstation idle or no extra draw. |

| Transition origin | Suggested transition weights |
| --- | --- |
| From workstation_work | Stay workstation 70-85%; switch to soldering 5-15%; short absence 5-10%; coffee/consultation/idle 5-10%. |
| From soldering_work | Stay soldering 55-75%; return workstation 15-25%; short absence/coffee 5-15%. |
| Young professor modifier | Increase soldering transition weight; increase late/weekend continuation; reduce early departure probability. |

# 8. Student class events

Student classes are fixed scheduled events, not isolated random toggles.

Default schedule: two classes per week, each 1.5 hours, six students, all six soldering workplaces reserved.

Example schedule: Tuesday 14:00-15:30 and Thursday 14:00-15:30. This should be configurable.

During class, professors do not independently occupy soldering workplaces unless modeled as a short supervisor demonstration.

| Lesson subtype | Probability | Device activation ranges |
| --- | --- | --- |
| debugging_session | 15% | Laptops 80-100%; lab PSUs 50-80%; soldering stations 10-30%; hot glue rare; heat gun almost never. |
| electronics_practice | 50% | Laptops 40-80%; lab PSUs 60-90%; soldering stations 40-70%; hot glue 5-15%; heat gun 2-5% per lesson. |
| soldering_practice | 35% | Soldering stations 80-100%; soldering lamps high; lab PSUs 30-60%; laptops 20-50%; hot glue 10-25%; heat gun 5-10% per lesson. |

# 9. Random student visit events

| Parameter | Suggested value / rule |
| --- | --- |
| Weekday visitor-day probability | 35-60% chance that at least one random student visit happens. |
| Weekend visitor-day probability | 5-20%, mostly if young professor/lab access is plausible. |
| Visitor count | 1-3 students. |
| Visit duration | 1-8 hours for normal visits; 15-60 minutes for quick_visit. |
| Supervision/access | Consultation requires professor present. Unsupervised soldering should be impossible or very rare. |

| Visit activity | Probability | Devices |
| --- | --- | --- |
| laptop_debugging | 35% | Laptop, possible low-power lab PSU, possible phone charger. |
| soldering_project | 30% | Soldering station, soldering lamp, lab PSU, hot glue, rare heat gun/drill. |
| consultation | 20% | Low load; laptop/phone possible; requires professor. |
| quick_visit | 15% | Usually no equipment; phone charger possible. |

# 10. Shared event families and probability ranges

| Event | Trigger | Power/duration | Base probability / count |
| --- | --- | --- | --- |
| Kettle boil | At least one person present; usually professor group. | 1200 W for 3-6 min. | 0 people: 0. 1 professor: 0-1/day. 2+ professors: 1 boil 15%, 2 boils 50%, 3 boils 30%, 4 boils 5%. Class may add +1 with 20-35%. |
| Phone charging | Per present person, longer stays/classes. | 5-12 W typical, 20 W peak, 20-120 min. | More likely during long visits/classes; increased before/after outages. |
| Desk lamps | Person working + dark/weather condition. | 6 W binary. | See lighting section. |
| Ceiling lamps | People present + dark condition. | 40 W each. | 1 lamp if dark workstation work; 1-2 if soldering/equipment; disabled/reduced during outages. |
| Hot glue gun | Soldering/hardware work. | 20-60 W cycling, 2-10 min. | Soldering class 10-25%; student project/professor work lower. |
| Heat gun | Soldering/hardware work only. | 1500 W for 1-5 min. | Professor soldering 2-6%; class 5-10%; student project 2-5%; heavily suppressed during outages. |
| Hand drill | Maintenance/hardware assembly. | 400 W, peak 450 W, 10-90 sec. | Professor session 2-5%; student project 1-3%; class 0-2%. |

# 11. Lighting behavior

Lighting is behavior-driven and environment-sensitive. Lamps do not turn on because of time alone; they turn on because a person is working and lighting conditions make it plausible.

| Condition | Workstation desk lamp probability | Soldering desk lamp probability | Ceiling lamp rule |
| --- | --- | --- | --- |
| Clear daylight | 5-15% | 15-30% | Usually off. |
| Cloudy/rain/fog | 35-65% | 60-90% | 1 lamp possible if several people present. |
| Normal day, 1 h before sunset | Probability increases. | Probability increases. | 1 lamp possible. |
| Cloudy/rainy day, 2 h before sunset | Probability increases earlier. | Probability increases earlier. | 1-2 lamps if soldering active. |
| After sunset | 70-95% | 90-100% | 1 lamp for workstation-only; 1-2 if soldering active. |

# 12. Grid and outage behavior modifiers

Grid/outage state is an input to behavior. It changes what people choose to turn on. The load module still returns only total_power_draw_w.

| Grid awareness state | Condition | Behavior modifiers |
| --- | --- | --- |
| grid_normal | Grid available and no near outage. | Base event probabilities. 2 kW inverter limit is ignored because pass-through mode applies. |
| planned_outage_soon | Outage starts within next 1-2 hours. | New soldering event x0.6; heat gun x0.3; laptop plug probability x1.2 while grid is still available; phone charging x1.3; people avoid starting long equipment-heavy tasks. |
| outage_active | Grid unavailable now. | Behavior depends strongly on SoC. People reduce optional load, avoid ceiling lamps/high power, and reduce laptop charging. |
| post_outage_recovery | Grid returned within last 30-60 min. | Phone charging x1.4; laptop plug probability x1.3; delayed low/medium work resumes; high-power events still reduced if another outage is soon. |

# 13. Battery SoC behavior modifiers

SoC affects behavior mainly during an active outage or when an outage is close. If grid is available and no outage is near, SoC should not strongly reduce normal lab consumption.

| SoC band | During outage / outage soon behavior |
| --- | --- |
| >= 70% comfortable | Desk lamps allowed; one ceiling lamp allowed only if soldering/dark; soldering probability x0.6; laptop plug x0.7; kettle x0.2; heat gun x0.05; ceiling lamps max 1. |
| 50-70% cautious | Workstation work allowed; soldering x0.3; laptop plug x0.5; kettle x0.05; heat gun 0; ceiling lamp only for active soldering + dark, max 1. |
| 30-50% restricted | Limited workstation/laptop work; desk lamps only if needed; no ceiling lamps; no kettle; no heat gun; soldering x0.1; laptop plug x0.25. |
| 15-30% critical | Only internet + one laptop/monoblock + minimal lamp if needed; new workstation x0.2; soldering 0; laptop plug x0.1; kettle/heat gun/drill/ceiling lamps 0. |
| < 15% emergency | People avoid using lab power. Only always-on critical internet baseline, except extremely rare minimal short event if desired. |

# 14. One-minute power calculation rules

At every simulated minute, start from critical internet baseline.

Resolve all active human presence and behavior events for that minute.

Activate devices required by those events.

Apply environment modifiers during event generation and activation decisions.

Sum power of all active devices for that minute.

For short events with duration below 1 minute, include their energy contribution in the minute by averaging over that minute. Do not ignore short events.

For 15-minute persisted rows, store both momentary_power_w at the timestamp and energy_wh_last_15m accumulated from one-minute samples.

| Device/event type | Minute-level behavior |
| --- | --- |
| Soldering station | Heat-up phase 60-75 W for 3-8 min; holding phase 15-35 W cycling. At 1-minute cadence this can be modeled directly. |
| Lab PSU | Usually 5-30 W; rare 30-100 W; very rare up to 200 W. |
| Hot glue gun | 20-60 W cycling, 2-10 min. |
| Phone charger | 5-12 W typical, 20 W peak, 20-120 min. |
| Kettle | 1200 W for 3-6 min. At 1-minute cadence it appears as a clear spike. |
| Heat gun | 1500 W for 1-5 min. Rare but visible spike. |
| Drill | 400 W active, 450 W peak, 10-90 sec. Average sub-minute duration into the minute if needed. |

# 15. Determinism and seeding

Same date range + same seed must generate the same professor holidays, illnesses, field work, classes, student visits, events and load points.

Use stable sub-seeds per year/date/agent/event family to avoid reshuffling old history when new future dates are generated.

Long-term backfill should not duplicate timestamps and should be gap-fill friendly.

Changing seed is allowed to produce a different but still plausible scenario for testing.

# 16. Suggested storage fields

| Field | Description |
| --- | --- |
| timestamp_utc / timestamp_local | Timestamp of the stored row. Long history rows are every 15 minutes; recent buffer may retain 1-minute points. |
| momentary_power_w | Power draw at that exact timestamp/minute. |
| energy_wh_last_15m | Total energy consumed over the previous 15 minutes. Required for future battery drain calculations. |
| active_professors_count | Optional debug/dashboard field. |
| active_students_count | Optional debug/dashboard field. |
| active_event_tags_json | Optional compact tags such as class, soldering, kettle, outage_active, dark_condition. |
| top_device_groups_json | Optional compact breakdown for explanation/debugging. |
| seed | Optional provenance/debug field. |

# 17. Acceptance tests

No people present and no events: total power equals critical internet baseline only.

If a professor is ill/holiday/field_work on a date, their workstation and personal events are inactive for that date.

Each professor uses exactly 14 holiday days per simulated year; split mode can be one block or 3-4 smaller breaks totaling 14 days.

Scheduled student classes occur twice per week for 1.5 hours and reserve all 6 soldering workplaces.

Kettle events never occur without people present and use 1200 W for 3-6 minutes.

Grid available state does not apply the 2 kW inverter limit; total_power_draw_w may exceed 2000 W.

Active outage with low SoC strongly suppresses high-power events, ceiling lamps, and laptop charging through behavior probabilities.

One-minute simulation captures short spikes; 15-minute rows correctly include energy_wh_last_15m.

Same seed and date range produce identical load history.

Sub-minute frontend jitter/interpolation does not affect stored simulation data.

# 18. Implementation guidance for Codex

Implement device definitions first as a static config/module.

Implement deterministic calendar generation for professor absence/presence and student scheduled classes.

Implement event generation at 1-minute cadence or event interval resolution and resolve it into 1-minute power points.

Implement aggregation to 15-minute persisted rows with momentary_power_w and energy_wh_last_15m.

Do not implement controller decisions inside this module.

Do not use grid/SoC to clip final load directly. Use grid/SoC to modify human/device event probabilities only.

Add tests for edge cases before adding frontend visualization.

End of feature request