SmartEnergy Lab Battery Simulation Module

Backend Feature Request / Implementation Specification

Project: bachelor thesis software module for a PV + battery laboratory stand

# 1. Purpose and scope

This document defines the backend battery simulation requirements for the SmartEnergy Lab project. The battery module is intentionally simpler than the load, solar, and grid modules. It should model the battery as an energy buffer with chemistry-specific usable capacity, voltage display curve, charge/discharge limits, and linear cycle-based degradation.

The model is designed for a thesis demonstration and for interaction with later EMS/charging logic. It must be explainable, deterministic enough for repeatable backfills/tests, and should avoid detailed electrochemical modeling.

# 2. Non-goals

No detailed electrochemical cell model.

No internal resistance, temperature model, Peukert correction, voltage sag, BMS balancing, or cell-level series/parallel simulation.

No real CC/CV charge-controller implementation inside the battery model.

No decision logic about whether to charge from grid or solar. That belongs to the EMS/charging module.

No frontend work in the initial backend implementation step.

No storage/backfill/API implementation until the pure battery engine is confirmed by tests and diagnostic plots.

# 3. YAML configuration contract

The user-facing station configuration should expose only the following battery fields:

battery:
  chemistry: lifepo4        # lead_acid | lifepo4 | li_ion
  nominal_voltage_v: 24     # allowed values: 12 or 24 only
  capacity_ah: 100          # nominal manufacturer capacity in ampere-hours
  installation_date: 2026-01-01

Validation rules:

chemistry must be one of: lead_acid, lifepo4, li_ion.

nominal_voltage_v must be exactly 12 or 24. No arbitrary voltage values.

capacity_ah must be positive.

installation_date must be a valid date.

All other battery behavior parameters are derived from built-in chemistry profiles, not configured by the user.

# 4. Chemistry profiles

The chemistry profile converts nominal manufacturer capacity into safe usable simulation capacity. Depth of discharge is treated as the fraction of nominal capacity removed from a full battery. Commercial battery cycle-life is strongly affected by DoD, and lithium batteries generally tolerate deeper discharge than lead-acid batteries [1], [2].

| Chemistry | Profile DoD | Operational usable fraction | Reference cycle life | Max charge C-rate | Charge efficiency | Discharge efficiency | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| lead_acid | 0.50 | 0.40 | 500 | 0.20C | 0.85 | 0.85 | Conservative VRLA/deep-cycle style profile. Normal operation avoids deep discharge. |
| lifepo4 | 1.00 | 0.80 | 3000 | 0.50C | 0.95 | 0.95 | Flat voltage profile, high cycle life, safe usable window. |
| li_ion | 0.90 | 0.72 | 800 | 0.50C | 0.93 | 0.93 | Generic lithium-ion pack profile, deeper usable DoD than lead-acid but lower cycle life than LiFePO4. |

Operational usable fraction is intentionally not the same as nominal capacity. It represents a safe working window. For example, a 24 V, 100 Ah LiFePO4 battery has nominal energy of 2400 Wh and an operational usable energy of 1920 Wh.

# 5. Core formulas

nominal_energy_wh = nominal_voltage_v * capacity_ah
profile_usable_energy_wh = nominal_energy_wh * profile_usable_dod_fraction
operational_usable_energy_wh = nominal_energy_wh * operational_usable_fraction
current_usable_capacity_wh = operational_usable_energy_wh * (soh_percent / 100)
soc_percent = energy_wh / current_usable_capacity_wh * 100

Initial state on installation date:

Battery is new: soh_percent = 100.

Battery starts at 50% of current usable capacity.

energy_wh = current_usable_capacity_wh * 0.50.

equivalent_cycles_today and total_equivalent_cycles start at 0.

# 6. Per-step battery update model

The pure battery engine receives requested battery power from a later EMS/charging module. It does not decide charging policy itself. The recommended backend interface is net battery power, where positive means charging and negative means discharging.

input per step:
  timestamp
  net_battery_power_w
  dt_minutes = 1 by default

if net_battery_power_w > 0:
    requested_charge_w = net_battery_power_w
    applied_charge_w = min(requested_charge_w, max_charge_power_w, empty_capacity_wh / (dt_hours * charge_efficiency))
    energy_wh += applied_charge_w * charge_efficiency * dt_hours

elif net_battery_power_w < 0:
    requested_discharge_w = abs(net_battery_power_w)
    applied_discharge_w = min(requested_discharge_w, energy_wh * discharge_efficiency / dt_hours)
    energy_wh -= applied_discharge_w / discharge_efficiency * dt_hours

energy_wh = clamp(energy_wh, 0, current_usable_capacity_wh)

The model should output applied power, not only requested power, because the battery may be full, empty, or charge-rate limited.

# 7. Battery state output

| Field | Meaning |
| --- | --- |
| timestamp_local / timestamp_utc | Time of the state point. |
| soc_percent | State of charge within current usable capacity, 0-100%. |
| soh_percent | State of health, starts at 100%, degrades linearly by equivalent cycles. |
| energy_wh | Current stored usable energy. |
| current_usable_capacity_wh | Operational usable energy after SoH degradation. |
| nominal_energy_wh | nominal_voltage_v * capacity_ah. |
| voltage_v | Approximate visual voltage from chemistry curve and SoC. Not used to compute SoC. |
| requested_power_w | Input requested net battery power. Positive charge, negative discharge. |
| applied_power_w | Actually applied power after battery limits. Positive charge, negative discharge. |
| status | idle, charging, discharging, full, empty, charge_limited, discharge_limited. |
| equivalent_cycles_today | Accumulated daily discharged energy divided by current usable capacity. |
| total_equivalent_cycles | Lifetime equivalent full cycles in the model. |

# 8. Cycle counting and degradation

Cycle degradation should be linear and equivalent-cycle based. This is a simplification: real cycle life depends on temperature, current, SoC range, and exact DoD [3]. For this project, those effects are intentionally ignored.

discharged_wh_today += discharge_energy_from_battery_wh
equivalent_cycles_today = discharged_wh_today / current_usable_capacity_wh

degradation_per_equivalent_cycle_percent = 20.0 / reference_cycle_life_to_80_percent
soh_loss_today_percent = equivalent_cycles_today * degradation_per_equivalent_cycle_percent
soh_percent -= soh_loss_today_percent

Daily degradation rule:

Apply degradation at the end of each local day.

After degradation, recompute current_usable_capacity_wh.

Clamp energy_wh to the new current_usable_capacity_wh if capacity decreased.

Reset equivalent_cycles_today and discharged_wh_today for the next local day.

Example: two 40% discharge-and-recharge events in one day equal 0.8 equivalent cycles.

# 9. Voltage display model

Voltage is a visual/live-feed estimate only. The battery SoC must be calculated from Wh integration, not from voltage. This is especially important for LiFePO4, whose discharge curve is relatively flat and makes voltage a weak SoC indicator [4].

voltage_v = nominal_voltage_v * interpolate_voltage_factor(chemistry, soc_percent)

Recommended approximate normalized voltage-factor curves:

| SoC % | lead_acid | lifepo4 | li_ion |
| --- | --- | --- | --- |
| 0 | 0.90 | 0.88 | 0.81 |
| 10 | 0.92 | 0.94 | 0.88 |
| 20 | 0.94 | 0.975 | 0.92 |
| 30 | 0.965 | 0.99 | 0.96 |
| 40 | 0.98 | 0.995 | 0.98 |
| 50 | 1.00 | 1.00 | 1.00 |
| 60 | 1.02 | 1.015 | 1.02 |
| 70 | 1.03 | 1.03 | 1.04 |
| 80 | 1.04 | 1.045 | 1.065 |
| 90 | 1.05 | 1.06 | 1.09 |
| 100 | 1.06 | 1.12 | 1.14 |

These curves are intentionally approximate and should be documented as dashboard curves. They do not model voltage sag, load current, battery relaxation, or temperature.

# 10. Integration boundaries

Battery engine responsibilities:

Parse/receive a validated battery config.

Derive chemistry profile parameters.

Update battery energy, SoC, voltage display, SoH, and equivalent cycles from requested battery power.

Clamp charge/discharge by capacity and max charge C-rate.

Report applied battery power and state.

EMS/charging module responsibilities, not battery engine responsibilities:

Decide whether to charge from grid, solar, or not charge.

Decide whether the load should be served from grid or battery.

Apply mode policies: solar priority, grid priority, backup preservation, etc.

Calculate requested net_battery_power_w based on solar/load/grid/controller decisions.

# 11. Recommended staged implementation

Stage B1: pure battery simulation engine and tests only. No DB/API/scheduler/frontend.

Stage B2: diagnostic battery probe with a few controlled charge/discharge scenarios and matplotlib charts.

Stage B3: integrate with load + grid + solar through a temporary EMS driver for visual validation.

Stage B4: only after behavior is confirmed, add storage/backfill/API if still needed.

# 12. Required tests for the pure battery engine

Config validation accepts only lead_acid, lifepo4, li_ion and 12/24 V.

Nominal energy and operational usable energy are calculated correctly.

Initial state is 50% SoC and 100% SoH at installation.

Charging increases energy and clamps at full capacity.

Discharging decreases energy and clamps at empty.

Charge power is limited by chemistry C-rate.

SoC is calculated from Wh balance, not from voltage.

Voltage output follows the selected chemistry curve by interpolation.

Equivalent cycles accumulate from discharged Wh.

Daily degradation reduces SoH by cycles * (20 / reference_cycle_life).

Energy is clamped if SoH degradation reduces usable capacity below current energy.

Battery engine does not import load, grid, solar generation, API, or storage modules.

# 13. Open decisions / notes for implementation

The exact YAML nesting should follow existing station config style. The logical user-facing fields remain chemistry, nominal_voltage_v, capacity_ah, and installation_date.

The first implementation should not attempt to model battery current, BMS, temperature, or internal resistance.

The reference profiles are generalized defaults and should be documented as such.

The voltage curve can be adjusted later if the frontend display needs better visual realism.

# References

[1] Depth of discharge. Wikipedia. Defines DoD and its relationship to SoC. https://en.wikipedia.org/wiki/Depth_of_discharge

[2] Rechargeable battery. Wikipedia. Summarizes typical lead-acid and lithium discharge limits. https://en.wikipedia.org/wiki/Rechargeable_battery

[3] Lithium-ion battery. Wikipedia. Notes cycle life is commonly specified to 80% capacity and depends on stress factors. https://en.wikipedia.org/wiki/Lithium-ion_battery

[4] Lithium iron phosphate. Wikipedia. Notes LiFePO4 has a flat discharge voltage curve and SoC-estimation challenges. https://en.wikipedia.org/wiki/Lithium_iron_phosphate

[5] Comparison of commercial battery types. Wikipedia. Provides broad cycle-life comparison for lead-acid, lithium-ion, and LiFePO4. https://en.wikipedia.org/wiki/Comparison_of_commercial_battery_types